# Aventro Motors: How to Change a Tyre

Changing a flat tyre is a basic and essential automotive skill for every Aventro Motor car owner. This guide will walk you through:

- Steps to change a tyre

- Where the spare tyre and toolkit is found in all Aventro models

- General tips and safety instructions

## Where to Find the Spare Tyre & Toolkit in Aventro Models

| Model Type | Common Location of Spare Tyre & Toolkit |
| --- | --- |
| Hatchback | Under the boot floor in the rear compartment |
| Sedan | Under the boot floor in the trunk |
| SUV | Under the boot floor or mounted underneath the rear of the car |
| (All Models) | Toolkit is usually secured next to the spare tyre or inside a side panel of the boot/trunk |

Note: All variants (Petrol/Diesel/Electric/Hybrid) follow the same compartment arrangement.

## Steps to Change a Tyre

### 1. Park Safely

- Pull over to a safe, flat location away from traffic.

- Turn on your hazard lights.

- Apply the handbrake.

- Place wheel wedges/blocks (if available) to prevent vehicle movement.

### 2. Gather Tools

You will need: - Spare tyre (found as per table above) - Jack (usually in toolkit) - Lug wrench (in toolkit) - Wheel wedges/blocks (optional) - Vehicle owner’s manual (for reference)

### 3. Remove the Wheel Cover/Hubcap

- If your wheel has a cover, use the flat end of the wrench or the provided lever to gently pry it off.

### 4. Loosen the Lug Nuts

- Use the lug wrench to turn the lug nuts counter-clockwise.

- Loosen them slightly – do not remove fully at this stage.

### 5. Lift the Vehicle with the Jack

- Position the jack under the jacking point nearest to the flat tyre. Refer to your owner’s manual for the exact point.

- Raise the vehicle until the flat tyre is about 6 inches above the ground.

### 6. Remove the Lug Nuts and Tyre

- Unscrew the lug nuts completely.

- Remove the flat tyre and set it aside.

### 7. Mount the Spare Tyre

- Line up the holes and push the spare onto the hub.

- Replace the lug nuts and tighten them by hand.

### 8. Lower the Vehicle

- Carefully lower the car with the jack until the tyre touches the ground firmly.

- Remove the jack.

### 9. Tighten the Lug Nuts

- Use the lug wrench to tighten the lug nuts in a star pattern to ensure even pressure.

### 10. Replace the Hubcap or Wheel Cover (if applicable)

- Place the hubcap back onto the wheel.

### 11. Stow Equipment

- Put the flat tyre, jack, and tools back in their compartments.

- Double check everything is secure.

## Safety & Tips

- Never change a tyre on a slope or close to traffic.

- Use hazard warning triangle if available.

- After installation, drive slowly and get the spare tyre pressure checked at the nearest Aventro Service Center.

- Some spare tyres are “space savers” and are not intended for long distances or high speeds.

## Need Help?

Aventro offers Roadside Assistance across India. If you are unable to change the tyre, please contact your nearest:

- Aventro Service Center

- Or call Aventro Roadside Assistance (see main service manual/contact card in your glove box)

## Find Service Centers

- Mumbai: Andheri East

- Delhi: Okhla Industrial Area, Phase II

- Bengaluru: Electronic City

- Chennai: Guindy Industrial Estate

- Hyderabad: Banjara Hills

- Pune: Wakad

- Kolkata: Salt Lake, Sector V

- Ahmedabad: Vastrapur

- Jaipur: Mansarovar

- … and many more across India!

For full list and directions, visit Aventro Motors Website.

Drive safe with Aventro Motors!