# How to Handle “Engine Won’t Start” Issue

When your Aventro vehicle’s engine refuses to start, it can be frustrating. Use this step-by-step guide to help diagnose and resolve the problem, or to gather information before contacting an Aventro Service Center.

## 1. Check the Basics

- Gear Position: Ensure the car is in ‘Park’ (automatic) or ‘Neutral’ (manual).

- Steering Lock: Gently turn the steering wheel while turning the key if steering is locked.

- Key/Start Button: Switch ignition off and retry. For push-button start, press brake/clutch while starting.

## 2. Battery Check

- Symptoms: No lights/display, slow/absent cranking sounds, clicking noise.

- Actions:

  - Inspect dashboard for dim or no lights.

  - Try using headlights or infotainment; if dim or off, battery may be the culprit.

  - If available, try jump-starting with a healthy battery.

  - For Electric variants: Ensure Battery Pack has adequate charge. Attempt Fast Charging if possible.

## 3. Fuel System Inspection

- Low Fuel/Empty Tank: Check fuel gauge and fill if necessary.

- Diesel Vehicles: Ensure there’s no air lock or recent filter change.

- Hybrid/Electric: Ensure main battery charge is sufficient.

## 4. Starter Motor & Electricals

- Symptoms: Single click or silence, even with good battery.

- Actions:

  - Check for blown fuses (refer to user manual location).

  - Test if interior lights stay bright when key is turned; if yes, suspect starter relay/motor.

## 5. Check Engine Warning Lights

- Look for: Engine/Malfunction lights on dashboard.

- Action: If warning lights appear, note down symbol/color for service personnel.

## 6. Listen for Sounds

- Rapid Clicking: Battery/starter issues.

- Engine Cranking (but not starting): Possible fuel, ignition, or electrical problems.

- No Sound at all: Battery, ignition switch, or starter.

## 7. Check Engine Components

- Loose Battery Terminals: Tighten if needed.

- Corroded Terminals: Clean carefully.

- Fuses: Replace if blown (carry spare fuses if possible).

- Relays: Swap with identical relays if available for testing.

- Security Systems: Deactivate aftermarket alarms/immobilizers, if installed.

## 8. Call Aventro Roadside Assistance

- If the car still won’t start after these checks, contact Aventro Roadside Assistance:

  - Location: Find nearest Aventro Service Center.

  - Helpline: +91 124 1234567