# How to Tow Your Aventro Car

Whether you’ve had a breakdown, a minor accident, or just need to move your Aventro vehicle, towing it safely is critical. Below is a step-by-step guide for both manual and automatic Aventro car models.

## Before You Begin

- Check Owner’s Manual: Refer to your specific model’s owner’s manual for any towing restrictions or recommendations.

- Warning! Never tow with wheels on the ground if your model is electric. Use a flatbed truck.

## Step 1: Gather Necessary Tools

- Tow strap or tow rope (rated for your car weight)

- Tow hook (provided with vehicle, usually in trunk/tool kit)

- Torch (for visibility at night)

- Warning triangle or cones

## Step 2: Locate the Tow Hook Covers

- Front and Rear Bumper:

  - Look for small plastic covers on the bumper (often marked by a tow symbol).

- Remove the Cover:

  - Use a plastic pry tool or key to open the cover.

## Step 3: Attach the Tow Hook

- Retrieve the Tow Hook:

  - Found in your car’s tool kit or trunk.

- Screw in the Tow Hook:

  - Insert into the exposed thread & rotate clockwise until tight.

## Step 4: Connect the Tow Strap or Tow Rope

- Attach to Tow Hook:
Ensure the strap/rope is securely fastened to the tow hook.

- Other Vehicle:
Attach the other end to the towing vehicle’s tow point.

## Step 5: Prepare the Vehicle for Towing

- Manual Transmission:
Place in Neutral, release the parking brake.

- Automatic Transmission:
Place in Neutral (N), release the parking brake.
CAUTION: For automatics and hybrids, flatbed towing is recommended to avoid transmission damage.

- Electric Vehicles:
ONLY FLATBED TOWING. Do not tow with wheels on ground.

## Step 6: Safety Measures

- Place warning triangle/cones at a safe distance behind both vehicles.

- Switch ON hazard lights on both vehicles.

- Ensure both drivers agree on signaling and communication.

## Step 7: Towing

- The towing vehicle should accelerate slowly – no sudden jerks.

- Maximum speed should not exceed 25 km/h.

- Keep the towed vehicle’s ignition ON (to prevent steering lock and allow brake assist).

- Steer and brake gently in the towed vehicle.

## Step 8: After Towing

- Park both vehicles safely.

- Unscrew the tow hook and store it back with your tools.

- Replace the plastic tow hook cover.

## Important Safety Notice

- Never tow on highways unless necessary.

- Do not allow passengers in the towed vehicle.

- If in doubt, use Aventro Roadside Assistance or professional towing.

## Contact Aventro Motors Roadside Assistance

- Phone: +91 124 1234567

- Email: info@aventromotors.com

For model-specific instructions, always refer to your Aventro Owner’s Manual.